#include <WiFi.h>

#include <WebServer.h>

#include <SPI.h>

#include <LoRa.h>

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

// NODE SETTINGS

#define NODE_ID 2

const char* AP_SSID = "Node 2";

const char* AP_PASSWORD = "11111111";

// OLED SETTINGS

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 32

#define OLED_RESET -1

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);

// LORA SETTINGS

#define LORA_SCK   5

#define LORA_MISO  19

#define LORA_MOSI  27

#define LORA_SS    18

#define LORA_RST   14

#define LORA_DIO0  26

#define LORA_FREQ  433E6

//LED

// LED SETTINGS

#define LED_PIN 23

// GLOBALS

WebServer server(80);

String statusMessage = "";

bool newMessageFlag = false;

#define MAX_MESSAGES 10

String msgHistory[MAX_MESSAGES];

int fromHistory[MAX_MESSAGES];

int msgCount = 0;

unsigned long displayTimer = 0;

bool displayActive = false;

// FUNCTION DECLARATIONS

String buildPage();

void handleRoot();

void handleSend();

void checkLoRaReceive();

void showOnDisplay(uint8_t from, uint8_t to, String msg);

void clearDisplaySafe();

void blinkSent();

void blinkReceived();

// SETUP

void setup() {

Serial.begin(115200);

delay(1000);

Serial.println();

Serial.println("BOOTING...");

pinMode(LED_PIN, OUTPUT);

digitalWrite(LED_PIN, LOW);

// WIFI

WiFi.mode(WIFI_AP);

bool apResult = WiFi.softAP(AP_SSID, AP_PASSWORD);

if(apResult){

Serial.println("WiFi AP Started");

Serial.println(WiFi.softAPIP());

} else {

Serial.println("WiFi AP FAILED");

}

// OLED

Serial.println("Initializing OLED...");

if(!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

Serial.println("OLED FAILED");

while(1);

}

Serial.println("OLED OK");

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0,0);

display.println("CampusConnect");

display.print("Node:");

display.println(NODE_ID);

display.println("");

display.print("URL:");

display.println(WiFi.softAPIP());

display.display();

Serial.println("OLED DISPLAYED");

// LORA

Serial.println("Initializing LoRa...");

SPI.begin(LORA_SCK, LORA_MISO, LORA_MOSI, LORA_SS);

LoRa.setPins(LORA_SS, LORA_RST, LORA_DIO0);

if (!LoRa.begin(LORA_FREQ)) {

Serial.println("LoRa FAILED");

while (1);

}

Serial.println("LoRa OK");

// WEB SERVER

server.on("/", handleRoot);

server.on("/send", handleSend);

server.on("/check", []() {

if (newMessageFlag) {

server.send(200, "text/plain", "1");

newMessageFlag = false;   // move AFTER send

} else {

server.send(200, "text/plain", "0");

}

});

server.begin();

Serial.println("SERVER STARTED");

Serial.println("SETUP COMPLETE");

}

// LOOP

void loop() {

server.handleClient();

checkLoRaReceive();

if (displayActive && millis() - displayTimer > 3000) {

clearDisplaySafe();

}

}

// WEB UI

String buildPage() {

String page =

"<!DOCTYPE html><html><head>"

"<meta charset='UTF-8'>"

"<meta name='viewport' content='width=device-width, initial-scale=1'>"

"<title>CampusConnect - Node 2</title>"

"<style>"

"body{margin:0;padding:0;font-family:Segoe UI,Roboto,sans-serif;background:#f4f6f8;color:#111827;}"

".wrapper{min-height:100vh;display:flex;align-items:center;justify-content:center;padding:16px;}"

".card{background:#fff;max-width:420px;width:100%;border-radius:14px;box-shadow:0 10px 25px rgba(0,0,0,0.1);padding:22px;}"

".title{font-size:20px;font-weight:600;margin-bottom:4px;}"

".badge{font-size:12px;background:#e0f2fe;color:#075985;padding:5px 12px;border-radius:999px;display:inline-block;margin-bottom:10px;}"

".subtitle{font-size:13px;color:#6b7280;margin-bottom:18px;}"

"label{font-size:13px;font-weight:500;display:block;margin-bottom:6px;}"

"input{width:100%;padding:11px;border-radius:10px;border:1px solid #d1d5db;margin-bottom:14px;font-size:14px;}"

"button{width:100%;padding:12px;border:none;border-radius:999px;background:#2563eb;color:white;font-weight:600;font-size:14px;cursor:pointer;}"

".status{margin-top:12px;padding:10px;border-radius:10px;background:#ecfdf5;color:#166534;font-size:13px;border:1px solid #bbf7d0;}"

".message-card{margin-top:12px;padding:12px;border-radius:12px;background:#f9fafb;border:1px solid #e5e7eb;}"

".message-meta{font-size:12px;color:#6b7280;margin-bottom:4px;}"

"table{width:100%;border-collapse:collapse;font-size:13px;margin-top:4px;}"

"th,td{border:1px solid #d1d5db;padding:6px;text-align:left;}"

"th{background:#e5e7eb;}"

"</style>"

"<script>"

"setInterval(()=>{fetch('/check').then(r=>r.text()).then(t=>{if(t==='1')location.reload();});},1000);"

"</script>"

"</head><body><div class='wrapper'><div class='card'>"

"<div class='title'>CampusConnect</div>"

"<div class='badge'>Node 2</div>"

"<div class='subtitle'>LoRa Messaging Interface</div>"

"<form action='/send'>"

// FIXED DESTINATION

"<label>Destination</label>"

"<input type='text' value='Node 1' disabled>"

"<input type='hidden' name='to' value='1'>"

// FIXED SENDER

"<label>Sender Name</label>"

"<input type='text' value='CampusRatty' disabled>"

"<input type='hidden' name='sender' value='CampusRatty'>"

"<label>Message</label>"

"<input type='text' name='msg' maxlength='190' required>"

"<button type='submit'>Send Message</button>"

"</form>";

if (statusMessage != "") {

page += "<div class='status'>" + statusMessage + "</div>";

}

// MESSAGE TABLE

if (msgCount > 0) {

page += "<div class='message-card'><div class='message-meta'>Recent Messages</div>";

page += "<table><tr><th>From Node</th><th>Message</th></tr>";

for (int i = msgCount - 1; i >= 0; i--) {

page += "<tr><td>" + String(fromHistory[i]) + "</td><td>" + msgHistory[i] + "</td></tr>";

}

page += "</table></div>";

}

page += "</div></div></body></html>";

return page;

}

void handleRoot() {

server.send(200, "text/html", buildPage());

}

// SEND

void handleSend() {

String msg = server.arg("msg");

int toNode = server.arg("to").toInt();

String senderName = server.arg("sender");

// SEND LORA PACKET

LoRa.beginPacket();

LoRa.write((uint8_t)toNode);

LoRa.write((uint8_t)NODE_ID);

LoRa.print("From " + senderName + ": " + msg);

LoRa.endPacket();

blinkSent();

// STATUS

statusMessage = "✅ Message sent from " + senderName;

// SAVE MESSAGE TO WEB HISTORY

if (msgCount < MAX_MESSAGES) {

fromHistory[msgCount] = NODE_ID;

msgHistory[msgCount] = "From " + senderName + ": " + msg;

msgCount++;

} else {

for (int i = 0; i < MAX_MESSAGES - 1; i++) {

fromHistory[i] = fromHistory[i + 1];

msgHistory[i] = msgHistory[i + 1];

}

fromHistory[MAX_MESSAGES - 1] = NODE_ID;

msgHistory[MAX_MESSAGES - 1] = "From " + senderName + ": " + msg;

}

// OLED

showOnDisplay(NODE_ID, toNode, msg);

// REDIRECT

server.sendHeader("Location", "/");

server.send(302);

}

// RECEIVE

void checkLoRaReceive() {

int packetSize = LoRa.parsePacket();

if (packetSize) {

uint8_t toNode = LoRa.read();

uint8_t fromNode = LoRa.read();

char incoming[256];

int index = 0;

while (LoRa.available() && index < 255) {

incoming[index++] = (char)LoRa.read();

}

incoming[index] = '\0';

String msg = String(incoming);

Serial.println("Received packet:");

Serial.println(msg);

if (toNode == NODE_ID) {

blinkReceived();

statusMessage = "📩 New message received!";

newMessageFlag = true;

// STORE MESSAGE

if (msgCount < MAX_MESSAGES) {

fromHistory[msgCount] = fromNode;

msgHistory[msgCount] = msg;

msgCount++;

} else {

for (int i = 0; i < MAX_MESSAGES - 1; i++) {

fromHistory[i] = fromHistory[i + 1];

msgHistory[i] = msgHistory[i + 1];

}

fromHistory[MAX_MESSAGES - 1] = fromNode;

msgHistory[MAX_MESSAGES - 1] = msg;

}

Serial.println("Stored successfully");

showOnDisplay(fromNode, toNode, msg);

}

}

}

// OLED

void showOnDisplay(uint8_t from, uint8_t to, String msg) {

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 0);

display.print("From : ");

display.print(from);

display.print(" To : ");

display.println(to);

display.println("");

display.println("Message Sent ");

// 16 chars per line fits nicely

display.display();

displayTimer = millis();

displayActive = true;

}

void clearDisplaySafe() {

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0,0);

display.println("CampusConnect");

display.print("Node:");

display.println(NODE_ID);

display.println("");

display.print("URL:");

display.println(WiFi.softAPIP());

display.display();

displayActive = false;

}

// BLINK ON SEND (1 blink)

void blinkSent() {

for(int i = 0; i < 2; i++) {

digitalWrite(LED_PIN, HIGH);

delay(100);

digitalWrite(LED_PIN, LOW);

delay(100);

}

}

// BLINK ON RECEIVE (2 fast blinks)

void blinkReceived() {

for(int i = 0; i < 4; i++) {

digitalWrite(LED_PIN, HIGH);

delay(100);

digitalWrite(LED_PIN, LOW);

delay(100);

}

}